| Lab Junior Spec |  |  |
| --- | --- | --- |
| Optical Entrance | Input Round | Diameter : 0.5 ㎜ |
|  |  | NA = 0.22 |
|  |  | monted in SMA-coupling |
| Slit | Output Linear | 50 ㎛ × 1000 ㎛ (standard) |
| Grating |  | Flat-field, |
|  |  | 600 Line/㎜ (center) |
|  |  | Blazed at 500 ㎚ (350 ~ 850 ㎚) |
| Spectral Range |  | 400 ㎚ ~ 800 ㎚ (Tungsten Halogen) |
| Dimension | Total (with case) | 35 × 98 × 250 ㎣ |
|  | Light (TH 2100) | 35 × 98 × 118 ㎣ |
|  | Spectrometer(SV2100) | 35 × 98 × 118 ㎣ |
|  | Cuvette Holder | 52 × 95 × 95 ㎣ |
| CCD Array | Producer | Sony |
|  | Number of Pixels | 2048 element |
|  | Dimensions of Pixels | 14 × 200 ㎛² per element |
|  | Maximum Clock-rate | 2 ㎒ |
|  | Current Consumption | 10 ㎃ |
|  | Supply Voltage | 5 V |
|  | Intefration Time | 1 ㎳ ~ 60 sec |
|  | Effective Range | 185 ~ 1100 ㎚ |
| Light Source - TH 2100 | Spectral Range | 360 ㎚ ~ 5 microns |
|  | Power Input | 9 V DC /3A |
|  | Color Temp. | 2,800 K |
|  | Bilb Life | 1,500 hours |
|  | Bilb Output | 70 Lumens |
|  | Connector | SMA 905 |
| Interface | Connector Assignment | USB 2.0 |